SIMATS ENGINEERING

ASSESSMENT TOOL 2

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA0713

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Annexure A

Scenario-Based

Code of Conduct:

I _jeevika.k (192571060)___________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

Annexure A

Scenario-Based

1. A newly established Smart Campus is deploying a secure network infrastructure for its academic and administrative departments. The network administrator has been allocated the IP address block 192.168.100.0/24. Based on the expected number of devices, the following subnet masks have already been assigned:

School of Computing → /25

School of Electronics → /26

School of Mechanical Engineering → /27

As the network engineer, design the subnet allocation by performing the following tasks:

- Determine the network address and broadcast address for each department.

- Identify the valid host IP address range for each subnet.

- Calculate the number of usable host addresses available in each subnet.

- Determine the remaining unused IP address range within the given network block.

2. A multinational software company is expanding its headquarters and has been assigned the private IP block 10.10.0.0/16. Different departments require different subnet sizes based on the number of employees. The network administrator has allocated the following subnet masks:

Software Development → /24

Quality Assurance → /25

Human Resources → /26

Executive Management → /27

As the network administrator, perform the following tasks:

- Determine the network address and broadcast address for each department.

- Identify the valid range of host IP addresses.

- Calculate the number of usable hosts in each subnet.

- Identify the unused IP address range remaining within the allocated IP block.

3.A smart hospital is migrating its network infrastructure from IPv4 to IPv6 to support thousands of connected medical devices and IoT sensors. The hospital has been assigned the IPv6 network:

2001:0db8:abcd:0000:0000:0000:0000:0000/64

As the network engineer responsible for the migration, perform the following:

- Convert the given IPv6 address into its compressed notation.

- Expand the compressed address back into its full hexadecimal representation.

- Identify the network prefix using the /64 prefix length.

- Calculate the total number of host addresses supported within the subnet.

- the total number of possible host addresses available within this network.

4. An Internet Service Provider (ISP) connects multiple customer networks through a core router. The router maintains the following routing table entries:

192.168.1.0/24

192.168.2.0/24

192.168.3.0/24

A packet arrives at the router with the destination IP address:

192.168.2.45

As the routing engineer, determine:

- The destination subnet to which the packet belongs.

- The matching routing table entry.

- The next-hop network/interface used to forward the packet.

- The default route that should be used if the destination does not match any routing table entry.

5. A company establishes two office departments connected through a Layer-3 router. The allocated subnet information is as follows:

Administration LAN → 192.168.10.0/26

Finance LAN → 192.168.10.64/26

The router interfaces are configured with:

Administration Gateway → 192.168.10.1

Finance Gateway → 192.168.10.65

As the network administrator, perform the following tasks:

- Determine the network address and broadcast address for each LAN.

- Identify the valid host IP address range.

- Suggest suitable IP addresses for three computers in each LAN.

- Identify the default gateway that hosts in each LAN should use for communication.

RUBRICS FOR CALCULATION

| Scenario Based Assessment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightage (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| Understanding of Scenario | 20% | Demonstrates clear and complete understanding of the scenario and context | Shows good understanding with minor gaps | Partial understanding; misses key aspects | Misinterprets or fails to understand the scenario |
| Identification of Key Issues | 20% | Accurately identifies all relevant issues and factors involved | Identifies most key issues with minor omissions | Identifies limited issues; some irrelevant points | Unable to identify key issues |
| Application of Concepts | 25% | Applies appropriate concepts effectively to address the scenario | Applies concepts with minor errors | Limited or partially correct application | Unable to apply relevant concepts |
| Decision Making | 20% | Makes well-reasoned, logical decisions with strong rationale | Makes reasonable decisions with some justification | Makes weak decisions with limited justification | No clear decisions made or rationale provided |
| Clarity of Response | 15% | Response is clear, structured, and logically presented | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganized and difficult to understand |

# Scenario-Based Answers

# 1. Smart Campus Network (192.168.100.0/24)

## Given

- Network: 192.168.100.0/24

- School of Computing → /25

- School of Electronics → /26

- School of Mechanical Engineering → /27

## (i) School of Computing (/25)

Subnet Mask

255.255.255.128

Network Address

192.168.100.0

Broadcast Address

192.168.100.127

Valid Host Range

192.168.100.1 – 192.168.100.126

Usable Hosts

2(32−25)−2=1262^{(32-25)}-2=126

126 Hosts

## (ii) School of Electronics (/26)

Next available subnet starts at 192.168.100.128

Subnet Mask

255.255.255.192

Network Address

192.168.100.128

Broadcast Address

192.168.100.191

Host Range

192.168.100.129 – 192.168.100.190

Usable Hosts

26−2=622^6-2=62

62 Hosts

## (iii) School of Mechanical (/27)

Next subnet starts at 192.168.100.192

Subnet Mask

255.255.255.224

Network Address

192.168.100.192

Broadcast Address

192.168.100.223

Host Range

192.168.100.193 – 192.168.100.222

Usable Hosts

25−2=302^5-2=30

30 Hosts

## (iv) Remaining Unused IP Range

Remaining block

192.168.100.224 – 192.168.100.255

### Final Table

| Department | Network | Broadcast | Host Range | Usable Hosts |
| --- | --- | --- | --- | --- |
| Computing | 192.168.100.0/25 | 192.168.100.127 | 192.168.100.1 – 126 | 126 |
| Electronics | 192.168.100.128/26 | 192.168.100.191 | 192.168.100.129 – 190 | 62 |
| Mechanical | 192.168.100.192/27 | 192.168.100.223 | 192.168.100.193 – 222 | 30 |

Unused Range

192.168.100.224 – 192.168.100.255

# 2. Multinational Company (10.10.0.0/16)

## (i) Software Development (/24)

Network

10.10.0.0

Broadcast

10.10.0.255

Host Range

10.10.0.1 – 10.10.0.254

Usable Hosts

254

## (ii) Quality Assurance (/25)

Next subnet

10.10.1.0

Broadcast

10.10.1.127

Host Range

10.10.1.1 – 10.10.1.126

Usable Hosts

126

## (iii) Human Resources (/26)

Network

10.10.1.128

Broadcast

10.10.1.191

Host Range

10.10.1.129 – 10.10.1.190

Usable Hosts

62

## (iv) Executive Management (/27)

Network

10.10.1.192

Broadcast

10.10.1.223

Host Range

10.10.1.193 – 10.10.1.222

Usable Hosts

30

## Remaining Unused Range

10.10.1.224 – 10.10.255.255

### Final Table

| Department | Network | Broadcast | Host Range | Hosts |
| --- | --- | --- | --- | --- |
| Software | 10.10.0.0/24 | 10.10.0.255 | 10.10.0.1–254 | 254 |
| QA | 10.10.1.0/25 | 10.10.1.127 | 10.10.1.1–126 | 126 |
| HR | 10.10.1.128/26 | 10.10.1.191 | 10.10.1.129–190 | 62 |
| Executive | 10.10.1.192/27 | 10.10.1.223 | 10.10.1.193–222 | 30 |

# 3. IPv6 Address

Given

2001:0db8:abcd:0000:0000:0000:0000:0000/64

### (i) Compressed Form

2001:db8:abcd::/64

### (ii) Expanded Form

2001:0db8:abcd:0000:0000:0000:0000:0000

### (iii) Network Prefix

2001:db8:abcd::/64

Network Portion

2001:db8:abcd:0000

Host Portion

0000:0000:0000:0000

### (iv) Number of Hosts

Host bits

128−64=64

Hosts

2642^{64}

= 18,446,744,073,709,551,616

### (v) Total Possible Hosts

2⁶⁴ = 18,446,744,073,709,551,616

# 4. Routing Table

Routing Table

192.168.1.0/24

192.168.2.0/24

192.168.3.0/24

Destination

192.168.2.45

### (i) Destination Subnet

192.168.2.0/24

### (ii) Matching Route

192.168.2.0/24

### (iii) Next-Hop Interface

Packet forwarded through the interface connected to

192.168.2.0/24

(For example: Interface G0/1 if connected.)

### (iv) Default Route

0.0.0.0/0

or

Default Gateway

Used when no routing table entry matches the destination.

# 5. Company Network

## Administration LAN

Subnet

192.168.10.0/26

Network Address

192.168.10.0

Broadcast

192.168.10.63

Host Range

192.168.10.1 – 192.168.10.62

Gateway

192.168.10.1

Three PCs

PC1 192.168.10.2

PC2 192.168.10.3

PC3 192.168.10.4

## Finance LAN

Subnet

192.168.10.64/26

Network

192.168.10.64

Broadcast

192.168.10.127

Host Range

192.168.10.65 – 192.168.10.126

Gateway

192.168.10.65

Three PCs

PC1 192.168.10.66

PC2 192.168.10.67

PC3 192.168.10.68

## Final Table

| LAN | Network | Broadcast | Host Range | Gateway |
| --- | --- | --- | --- | --- |
| Administration | 192.168.10.0/26 | 192.168.10.63 | 192.168.10.1 – 192.168.10.62 | 192.168.10.1 |
| Finance | 192.168.10.64/26 | 192.168.10.127 | 192.168.10.65 – 192.168.10.126 | 192.168.10.65 |

Suggested IPs

Administration: 192.168.10.2, 192.168.10.3, 192.168.10.4

Finance: 192.168.10.66, 192.168.10.67, 192.168.10.68

Top of Form

Bottom of Form