SIMATS ENGINEERING

ASSESSMENT TOOL 2

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA0706

COURSE OUTCOME COVERED

CO4: 
Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards

(BL2,BL6)

Assessment Tool Weightage: 30%

Annexure A

Industry Problem Solving Task

Code of Conduct:

I _jeevika karuppasamy (192571060)_ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

Annexure A

Industry Problem Solving Task

- A star topology is ideal for a 20-person startup because all devices connect to a central switch, making the network easy to manage and troubleshoot. At the core sits a Layer 2/Layer 3 managed switch (such as a Cisco Catalyst or Netgear ProSafe) with at least 24 ports to accommodate all workstations, printers, and VoIP phones with room to grow. Each device connects to the switch using Cat6 UTP cables, which support Gigabit speeds up to 100 meters — more than sufficient for any typical office floor plan. The switch uplinks to a router/firewall (such as a Cisco RV series or pfSense appliance) that handles internet access, DHCP, and network address translation. A Wi-Fi access point connected to the switch extends wireless coverage for laptops and mobile devices. The key advantage of this design is fault isolation: if one workstation's cable fails, only that single node goes offline, leaving all other 19 employees unaffected. Centralised cable management also simplifies future moves, adds, and changes.

- A bank demands near-zero downtime, making a full or partial mesh topology the right choice. In a full mesh, every core network device — routers, core switches, and servers — connects directly to every other device, so no single link or node failure can bring down the network. For a bank, a partial mesh is typically deployed at the core layer (connecting all core switches and firewalls to each other) while the distribution and access layers use redundant uplinks to at least two core switches. Dual enterprise-grade routers (such as Cisco ASR or Juniper MX series) provide BGP-based redundant internet connectivity. All links between core devices use fiber optic cables — either single-mode for long distances between buildings or multi-mode OM4 within the data center — because fiber is immune to electromagnetic interference, supports very high bandwidth, and is essential for maintaining low-latency financial transactions. Redundant power supplies, UPS systems, and RAID storage arrays further complement the mesh design. The justification is straightforward: when a link fails in a mesh, traffic automatically reroutes through alternative paths in milliseconds using protocols like OSPF or BGP, satisfying regulatory requirements for uptime and data integrity.

- For a small classroom lab with a tight budget, bus topology offers the lowest hardware cost because all computers share a single common communication line — the bus — eliminating the need for a central switch or hub. A single coaxial cable (RG-58, 50-ohm) or, in a modern variation, a single Cat5e/Cat6 cable daisy-chained through an inexpensive unmanaged hub runs along the length of the room. Each computer connects to the backbone using a T-connector and a BNC connector in a classic coaxial setup, with terminators (50-ohm) placed at both ends of the cable to absorb signals and prevent reflection. For a more contemporary low-cost implementation, a single inexpensive 8- or 16-port unmanaged switch replaces the terminators and uses Cat5e UTP patch cables to each workstation — achieving bus-like simplicity at minimal cost. The primary limitation is that the network is susceptible to collisions and a single cable break disrupts all connected machines, but for a supervised classroom environment with light usage (internet browsing, file sharing), this trade-off is entirely acceptable. No dedicated server is required; a teacher's PC can share resources using built-in Windows/Linux file sharing.

- A large university campus serves thousands of users across multiple buildings — lecture halls, labs, libraries, dormitories, and administration — making a single topology inadequate. A hybrid topology combines the best elements of multiple designs: a fiber-based ring or mesh at the campus backbone level for redundancy, a star topology within each building for manageability, and bus or tree structures within individual labs for cost-efficiency. At the campus core, high-speed fiber optic cables (single-mode, 10 Gbps or 40 Gbps) interconnect core routers and multilayer switches using a ring or partial mesh arrangement, ensuring that no single link failure isolates an entire building. Each building has its own distribution switch connected to the campus core via dual fiber uplinks. Within each building, a traditional star topology branches from the distribution switch to floor-level access switches, which then connect individual devices via Cat6 cables. This hierarchical three-layer model — core, distribution, and access — is the industry standard for enterprise and campus networks because adding a new building simply means installing a new distribution switch and running fiber to the core, with no disruption to existing infrastructure. Wireless access points throughout the campus connect via PoE (Power over Ethernet) to access switches, eliminating the need for separate power cabling. The hybrid design thus delivers redundancy at the backbone, simplicity at the building level, and scalability campus-wide.

- Designing a network for a company with multiple departments involves creating a structured and scalable architecture using switches and routers to ensure efficient communication and security. Each department (such as HR, Finance, Sales, and IT) is connected through dedicated Layer 2 switches that form the access layer, allowing devices like computers and printers to communicate within the same department. These switches are then connected to a central Layer 3 switch or router, which acts as the core layer and enables communication between different departments through inter-VLAN routing. Virtual LANs (VLANs) are configured to logically separate each department’s network, improving security and reducing broadcast traffic. A router is also used to connect the internal network to external networks such as the internet, often with firewall functionality for security. Redundant links and backup paths can be included to ensure reliability, while proper IP addressing and subnetting are applied for efficient network management. This hierarchical design improves performance, enhances security, and allows easy expansion as the company grows.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Problem Analysis | 15% | Clearly identifies and deeply analyzes the industry problem with all factors | Good understanding with minor gaps | Basic understanding; some key aspects missing | Poor or incorrect understanding of the problem |
| Solution Design | 20% | Innovative, practical, and industry-relevant solution | Effective solution with minor limitations | Partially suitable solution | Inappropriate or unclear solution |
| Technical Implementation | 20% | Fully functional, accurate, and well-executed implementation | Mostly functional with minor errors | Partially implemented solution | Incomplete or non-functional implementation |
| Use of Tools & Technologies | 10% | Appropriate and advanced use of relevant tools and technologies | Correct use of tools | Limited or basic use of tools | Incorrect or no use of tools |
| Problem-Solving Approach | 10% | Logical, systematic, and well-structured approach | Mostly logical approach | Somewhat disorganized approach | No clear approach |
| Innovation & Creativity | 10% | Highly creative and original solution | Some creativity | Limited originality | No creativity |
| Testing & Validation | 5% | Thorough testing with real-world scenarios | Adequate testing | Minimal testing | No testing |
| Documentation & Presentation | 5% | Clear, professional, and well-documented | Good documentation | Basic documentation | Poor or no documentation |
| Teamwork / Collaboration | 5% | Excellent coordination and contribution (if team-based) | Good collaboration | Limited participation | No collaboration or contribution |

# Annexure A – Industry Problem Solving Task

# 1. Design a Network Topology for a 20-Person Startup

A star topology is the most suitable network design for a startup with around 20 employees because it is reliable, easy to manage, and allows future expansion. In a star topology, every device is connected individually to a central network switch, which serves as the communication hub. Unlike bus or ring topologies, the failure of one cable or device does not affect the rest of the network, making it highly dependable for business operations.

At the center of the network, a 24-port Layer 2 or Layer 3 managed switch, such as a Cisco Catalyst or Netgear ProSafe switch, is installed. This switch provides enough ports for all employee computers, printers, IP phones, and other network devices while leaving additional ports for future growth. Each workstation is connected to the switch using Category 6 (Cat6) Unshielded Twisted Pair (UTP) cables, which support Gigabit Ethernet speeds of up to 1 Gbps over distances of 100 meters. Cat6 cables are affordable, easy to install, and provide excellent performance for office environments.

The switch is connected to a router or firewall, such as a Cisco RV Series router or a pfSense firewall appliance. The router connects the internal network to the internet and performs essential functions such as Dynamic Host Configuration Protocol (DHCP), Network Address Translation (NAT), firewall security, and internet traffic management. DHCP automatically assigns IP addresses to all devices, reducing manual configuration and simplifying network administration.

A wireless access point (Wi-Fi AP) is also connected to the central switch to provide wireless internet connectivity for laptops, smartphones, and tablets. Modern Wi-Fi standards such as Wi-Fi 6 offer high-speed wireless communication while maintaining compatibility with existing devices.

One of the biggest advantages of the star topology is fault isolation. If the cable connected to one employee's computer becomes damaged, only that computer loses connectivity, while all other employees continue working normally. Troubleshooting is also easier because the administrator only needs to identify the faulty cable or port rather than examining the entire network.

Another important advantage is scalability. As the startup grows, new computers or devices can simply be connected to the available switch ports without disrupting the existing network. Network performance is also better because each device has a dedicated communication link to the switch, reducing collisions and improving data transmission efficiency.

Overall, the star topology provides excellent reliability, easy maintenance, high performance, and flexibility. These advantages make it the ideal networking solution for a 20-person startup where business continuity and future expansion are important.

Internet

|

Router / Firewall

|

24-Port Switch

---------------------------------

|   |   |   |   |   |   |   |   |

PC1 PC2 PC3 PC4 PC5 PC6 ... PC20

|                 |

Printer          Wi-Fi Access Point

# 2. Recommend a Network Topology for a Bank Requiring Near-Zero Downtime

Banks require highly reliable networks because even a few seconds of downtime can interrupt financial transactions, online banking, ATM services, and customer operations. Therefore, a full mesh or partial mesh topology is the most suitable network architecture for banking environments.

In a full mesh topology, every core networking device—including routers, switches, and servers—is directly connected to every other device. This creates multiple communication paths between devices. If one network link fails, data automatically travels through another available path without interrupting services. Although full mesh provides maximum redundancy, it becomes expensive and complex as the number of devices increases.

For this reason, most banks implement a partial mesh topology. In this design, only critical devices at the network core are fully interconnected, while distribution and access layer devices connect through redundant links. This approach provides excellent reliability while reducing installation costs.

Banks typically use enterprise-grade routers such as Cisco ASR or Juniper MX Series routers. These routers support advanced routing protocols like Border Gateway Protocol (BGP) for internet redundancy and Open Shortest Path First (OSPF) for dynamic internal routing. These protocols automatically detect network failures and reroute traffic within milliseconds.

The backbone network uses fiber optic cables because they offer extremely high bandwidth, low latency, and immunity to electromagnetic interference. Single-mode fiber is used for long-distance communication between buildings or branches, while multimode OM4 fiber is commonly used inside data centers. Fiber optics can support transmission speeds of 10 Gbps, 40 Gbps, or even 100 Gbps, making them suitable for modern banking applications.

To improve availability, banks also deploy dual power supplies, Uninterruptible Power Supply (UPS) systems, backup generators, RAID storage arrays, clustered servers, and redundant firewalls. These technologies ensure continuous operation even during equipment failures or power outages.

Security is another major requirement. Banks use Virtual LANs (VLANs), Access Control Lists (ACLs), firewalls, Intrusion Detection Systems (IDS), Intrusion Prevention Systems (IPS), encryption, and Virtual Private Networks (VPNs) to protect sensitive customer data and financial transactions.

The greatest advantage of the mesh topology is high fault tolerance. Since multiple communication paths exist, the failure of one cable, router, or switch does not affect network availability. Automatic rerouting maintains uninterrupted communication and satisfies strict banking regulations regarding system uptime and data integrity.

In conclusion, a partial mesh topology combined with fiber optic infrastructure, redundant hardware, and advanced routing protocols provides the reliability, security, and performance required for modern banking networks.

Internet

/             \

Router A ------- Router B

|\             /|

| \           / |

|  \         /  |

Core Switch 1---Core Switch 2

|\             /|

| \           / |

Server 1      Server 2

# 3. Recommend a Network Topology for a Small Classroom Laboratory

A bus topology is one of the most economical networking solutions for a small classroom laboratory where the budget is limited and network traffic is relatively low. In a bus topology, all computers share a single communication cable called the backbone cable, allowing devices to communicate over one common transmission medium.

The traditional implementation uses RG-58 coaxial cable with BNC connectors and T-connectors. Each computer connects to the backbone through a T-connector, while 50-ohm terminators are installed at both ends of the cable to prevent signal reflection. Without these terminators, communication errors would occur due to reflected electrical signals.

Although traditional bus networks are rarely used today, a modern low-cost variation uses an unmanaged Ethernet switch with Cat5e or Cat6 UTP cables. This design offers many of the cost benefits of a bus network while improving reliability and installation simplicity. An 8-port or 16-port unmanaged switch is affordable and requires minimal configuration.

One major advantage of the bus topology is low installation cost. Because fewer cables and networking devices are required, it is an attractive option for educational institutions with limited funding. Maintenance costs are also relatively low, and no dedicated server is required. The teacher's computer can act as a file-sharing server using Windows or Linux operating system features.

The network supports common classroom activities such as internet browsing, online learning, assignment submission, printing, and file sharing. Since only a small number of users access the network simultaneously, overall performance remains acceptable.

However, the bus topology has several disadvantages. Since all devices share the same communication medium, data collisions can occur when multiple computers attempt to transmit data simultaneously. As the number of users increases, network performance decreases significantly.

Another disadvantage is that the backbone cable represents a single point of failure. If the cable becomes damaged or disconnected, the entire network stops functioning. Troubleshooting can also be difficult because identifying the exact location of the cable fault requires careful inspection.

Despite these limitations, the bus topology remains suitable for a classroom laboratory where budget constraints are more important than high performance or redundancy. Its low cost, simple installation, and sufficient performance for light educational use make it an appropriate networking solution.

[Terminator]==============================[Terminator]

|         |         |        |        |

PC1       PC2       PC3      PC4      PC5

# 4. Design a Network Topology for a Large University Campus

A large university campus consists of multiple buildings, including lecture halls, libraries, laboratories, hostels, administration offices, and research centers. Because of its size and complexity, a hybrid topology is the most suitable networking solution.

A hybrid topology combines different network topologies to take advantage of their individual strengths. At the campus backbone level, a ring or partial mesh topology using high-speed fiber optic cables provides redundancy and fault tolerance. Within each building, a star topology connects users to access switches, while tree structures organize multiple switches hierarchically.

The campus backbone consists of core routers and multilayer switches connected using single-mode fiber optic cables supporting speeds of 10 Gbps, 40 Gbps, or higher. Dual fiber links ensure that if one connection fails, traffic is automatically rerouted through another path.

Each building contains a distribution switch connected to the campus core through redundant fiber links. Distribution switches manage traffic entering and leaving each building while enforcing security policies and routing between VLANs.

Within buildings, access switches connect individual computers, printers, IP phones, wireless access points, and laboratory equipment using Cat6 Ethernet cables. The star topology simplifies maintenance because each device has its own dedicated connection.

Wireless connectivity is provided through Power over Ethernet (PoE) access points, allowing both power and data transmission through a single Ethernet cable. Students and staff can securely access the network from laptops, tablets, and smartphones.

The network follows the three-layer hierarchical model consisting of the Core Layer, Distribution Layer, and Access Layer. This architecture improves scalability, simplifies troubleshooting, and allows easy expansion whenever new buildings are constructed.

Network segmentation using Virtual LANs (VLANs) separates departments such as administration, faculty, students, laboratories, and libraries. Layer 3 switches perform inter-VLAN routing while maintaining network security and reducing broadcast traffic.

The hybrid topology also supports modern technologies such as cloud computing, online examinations, Learning Management Systems (LMS), IP surveillance cameras, VoIP communication, and smart classrooms.

Overall, the hybrid topology provides excellent scalability, redundancy, reliability, security, and performance, making it the industry-standard solution for large university campuses.

Campus Core

(Core Switch/Router)

/      |       \

/       |        \

Building A  Building B  Building C

|            |           |

Distribution   Distribution  Distribution

Switch         Switch        Switch

/|\            /|\           /|\

PCs APs       PCs APs      PCs APs

# 5. Design a Network for a Company with Multiple Departments

A company with multiple departments such as Human Resources (HR), Finance, Sales, Marketing, Customer Support, and Information Technology (IT) requires a structured network that provides efficient communication, security, and future scalability. The best approach is to use a hierarchical network design consisting of the Access Layer, Distribution Layer, and Core Layer.

At the Access Layer, each department has its own Layer 2 switch connecting employee computers, printers, scanners, IP phones, and other network devices. These switches provide high-speed local communication while reducing network congestion.

The access switches connect to a Layer 3 switch at the Distribution Layer. This multilayer switch performs inter-VLAN routing, allowing different departments to communicate securely while maintaining separate broadcast domains. Routing between VLANs improves performance and reduces unnecessary network traffic.

Each department is assigned a separate Virtual LAN (VLAN). For example, HR may use VLAN 10, Finance VLAN 20, Sales VLAN 30, Marketing VLAN 40, and IT VLAN 50. VLANs logically separate departments even when they share the same physical infrastructure. This improves security by preventing unauthorized access between departments.

The Core Layer consists of a high-performance router or multilayer switch that connects the internal network to external networks such as the internet. The router performs NAT, DHCP, VPN services, firewall protection, and internet traffic management.

Network security is strengthened using firewalls, Access Control Lists (ACLs), authentication servers, antivirus software, and intrusion detection systems. Sensitive departments such as Finance and HR receive stricter access controls than general departments.

To improve reliability, redundant switches, backup routers, multiple internet connections, and UPS systems are installed. If one device fails, another automatically takes over, minimizing downtime.

An organized IP addressing and subnetting scheme is also implemented. Each department receives its own IP subnet, making network administration, troubleshooting, and security management much easier.

The hierarchical architecture offers several advantages, including better performance, simplified troubleshooting, improved security, and easy scalability. As the company grows, additional departments or employees can be added by creating new VLANs and connecting more switches without redesigning the entire network.

In conclusion, a hierarchical network using Layer 2 switches, Layer 3 switches, VLANs, routers, firewalls, and redundant connections provides a secure, scalable, and high-performance solution for companies with multiple departments. This design is widely used in modern enterprise environments because it supports business growth while ensuring efficient and secure communication.

Internet

|

Router / Firewall

|

Layer 3 Core Switch

-------------------------------------

|         |          |              |

VLAN 10   VLAN 20    VLAN 30      VLAN 40

HR      Finance      Sales          IT

|          |           |            |

Layer 2    Layer 2     Layer 2      Layer 2

Switch      Switch      Switch       Switch

|          |           |            |

PCs       PCs        PCs         PCs