SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME:	CSA07 COURSE CODE: Computer Networks

COURSE OUTCOME COVERED

CO1: Apply the functions of OSI layer in enabling reliable data transmission across networks. (BL2, BL3)

Assessment Tool Weightage: 40%

Annexure A

SHORT ANSWER TEST

Dr.V.Parthipan

Code of Conduct:

I _jeevika.k_____(192571060)______________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

Annexure A

Short Answer Test

1. A company needs to transfer a 150 MB design file from a client computer to a remote server over a network with a bandwidth of 50 Mbps. The Transport Layer divides the file into 1500-byte segments, while the Data Link Layer encapsulates each segment into frames by adding 40 bytes of header and trailer. Calculate the total number of segments and frames transmitted, the total transmission overhead introduced by the Data Link Layer, and estimate the total transmission time (ignoring propagation delay). Explain how the Transport and Data Link layers ensure reliable communication.

2. A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

3. A 12,000-byte IP packet is transmitted through a router where the Maximum Transmission Unit (MTU) is 1500 bytes. Assuming each fragment requires a 20-byte IP header, calculate the total number of fragments created and the total header overhead introduced. Explain how the Network Layer ensures successful delivery of fragmented packets to the destination.

4. A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

5. A multimedia application transfers a 600 MB video file. The Session Layer inserts a synchronization checkpoint after every 60 MB of transmitted data. During transmission, a failure occurs after 390 MB has been transferred. Determine the number of checkpoints created before failure and calculate the amount of data that must be retransmitted after recovery. Explain the importance of synchronization in the Session Layer.

6. Before transmission, the Presentation Layer compresses a 900 MB file by 40%. The compressed data is encrypted, adding 5% overhead to the compressed size. Calculate the compressed file size, the encrypted file size, and the total percentage reduction compared to the original file. Explain how the Presentation Layer improves transmission efficiency and security.

7.An FTP application transfers 3 GB of data over a network with an effective throughput of 120 Mbps. The Application Layer divides the data into requests of 3 MB each. Calculate the total number of requests generated and estimate the total transmission time in seconds. Discuss how the Application Layer provides reliable services to users.

8.A packet experiences the following processing delays at different OSI layers: Application Layer = 4 ms, Transport Layer = 3 ms, Network Layer = 5 ms, Data Link Layer = 2 ms, and Physical Layer = 1 ms. The propagation delay is 18 ms, and the transmission delay is 12 ms. Calculate the total end-to-end delay experienced by the packet and explain how each layer contributes to the communication process.

9.A network transfers 80 MB of useful data using frames of 1600 bytes, where each frame contains 80 bytes of protocol overhead added by different OSI layers. Calculate the total number of frames required, the total overhead transmitted, and the transmission efficiency as a percentage. Explain how protocol overhead affects network performance.

10.A hospital transfers a patient's medical record of 240 MB through an IoT-enabled healthcare network. The Presentation Layer compresses the data by 30%, the Transport Layer divides the compressed data into 1400-byte segments, and the Data Link Layer adds 60 bytes of overhead to each frame. Calculate the compressed file size, the total number of segments transmitted, the total protocol overhead introduced, and the final amount of data transmitted across the Physical Layer. Explain how multiple OSI layers work together to ensure secure and reliable communication.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 25% | Demonstrates a thorough, accurate understanding of concepts | Good understanding with minor gaps | Basic understanding with some misconceptions | Poor understanding; major misconceptions |
| Explanation | 25% | Detailed, well-explained answers with relevant examples | Clear explanation with adequate details | Limited explanation; lacks depth | Poor or unclear explanation |
| Application | 20% | Effectively applies concepts with strong analytical ability | Applies concepts with minor errors | Limited application with weak analysis | No application or incorrect analysis |
| Organization | 15% | Well-structured answers with logical flow and coherence | Organized with minor issues in flow | Basic structure, lacks clarity | Poor organization, incoherent answers |
| Accuracy | 10% | Completely accurate and covers all required points | Mostly accurate with minor omissions | Partially correct with missing points | Incorrect answers with major gaps |
| Clarity | 5% | Clear, neat, professional presentation | Understandable with minor issues | Limited clarity, readability issues | Poorly written, difficult to understand |

# 1. Transport Layer and Data Link Layer

### Given

- File size = 150 MB = 150,000,000 bytes

- Segment size = 1500 bytes

- Header + Trailer = 40 bytes/frame

- Bandwidth = 50 Mbps

### (i) Number of Segments

1500000001500=100000\frac{150000000}{1500}=100000

Segments = 100,000

Since one segment forms one frame,

Frames = 100,000

### (ii) Total Overhead

100000×40=4000000 bytes100000\times40=4000000\text{ bytes}

= 4 MB

### (iii) Total Data Transmitted

150000000+4000000=154000000 bytes150000000+4000000=154000000\text{ bytes}

Convert to bits

154000000×8=1232000000 bits154000000\times8=1232000000\text{ bits}

### (iv) Transmission Time

123200000050000000=24.64 seconds\frac{1232000000}{50000000}=24.64\text{ seconds}

### Answer

- Segments = 100,000

- Frames = 100,000

- Overhead = 4 MB

- Transmission Time = 24.64 s

### Reliability

- Transport Layer: Segmentation, sequencing, acknowledgements, retransmission, flow control.

- Data Link Layer: Framing, CRC error detection, MAC addressing.

# 2. Sliding Window

### Given

- Window Size = 6

- Total Frames = 48

### Number of Windows

48/6=848/6=8

Windows = 8

One frame lost in every window

### Retransmissions

8×1=88\times1=8

### Answer

- Windows = 8

- Retransmissions = 8

### Flow Control

Flow control prevents sender from transmitting faster than receiver can process data, avoiding congestion and improving efficiency.

# 3. IP Fragmentation

### Given

Packet = 12,000 bytes

MTU = 1500 bytes

IP Header = 20 bytes

Payload per fragment

1500−20=1480 bytes1500-20=1480\text{ bytes}

### Fragments

⌈12000/1480⌉=9\lceil12000/1480\rceil=9

### Header Overhead

9×20=180 bytes9\times20=180\text{ bytes}

### Answer

- Fragments = 9

- Header Overhead = 180 bytes

### Network Layer

- Fragmentation

- Routing

- Fragment offset

- Reassembly at destination

# 4. Sliding Window

Same as Question 2.

### Answer

- Windows = 8

- Retransmissions = 8

Flow control improves communication by preventing buffer overflow and maintaining smooth data transfer.

# 5. Session Layer

### Given

File = 600 MB

Checkpoint every 60 MB

Failure after 390 MB

### Checkpoints

60

120

180

240

300

360

Total checkpoints before failure

6

Last checkpoint = 360 MB

Retransmission

390−360=30 MB already sent after checkpoint390-360=30\text{ MB already sent after checkpoint}

Remaining data to send from checkpoint

600−360=240 MB600-360=240\text{ MB}

### Answer

- Checkpoints = 6

- Data retransmitted = 240 MB

### Importance

Synchronization checkpoints allow communication to resume from the last checkpoint instead of restarting the entire transmission.

# 6. Presentation Layer

### Given

Original = 900 MB

Compression = 40%

### Compressed Size

900×0.60=540 MB900\times0.60=540\text{ MB}

Encryption Overhead

540×0.05=27 MB540\times0.05=27\text{ MB}

Encrypted Size

540+27=567 MB540+27=567\text{ MB}

Reduction

900−567=333 MB900-567=333\text{ MB}

Percentage

333900×100=37%\frac{333}{900}\times100=37\%

### Answer

- Compressed Size = 540 MB

- Encrypted Size = 567 MB

- Reduction = 37%

### Presentation Layer

- Compression

- Encryption

- Data translation

# 7. FTP Application

### Given

Data = 3 GB = 3000 MB

Each Request = 3 MB

Throughput = 120 Mbps

### Requests

3000/3=10003000/3=1000

Convert Data

3000×8=24000 Mb3000\times8=24000\text{ Mb}

Time

24000/120=200 seconds24000/120=200\text{ seconds}

### Answer

- Requests = 1000

- Transmission Time = 200 s

### Application Layer

Provides FTP services, authentication, file management and user interaction.

# 8. End-to-End Delay

### Processing Delay

Application = 4

Transport = 3

Network = 5

Data Link = 2

Physical = 1

Total Processing

4+3+5+2+1=15 ms4+3+5+2+1=15\text{ ms}

Propagation Delay

18 ms

Transmission Delay

12 ms

Total Delay

15+18+12=45 ms15+18+12=45\text{ ms}

### Answer

Total End-to-End Delay = 45 ms

### Layer Contribution

- Application – User services

- Transport – Reliable delivery

- Network – Routing

- Data Link – Framing

- Physical – Bit transmission

# 9. Transmission Efficiency

### Given

Useful Data

80 MB

Frame Size

1600 bytes

Overhead

80 bytes

Payload

1600−80=1520 bytes1600-80=1520\text{ bytes}

Useful Data

80 MB

80,000,000 bytes80,000,000\text{ bytes}

Frames

⌈80000000/1520⌉=52632\lceil80000000/1520\rceil=52632

Overhead

52632×80=4210560 bytes52632\times80=4210560\text{ bytes}

Efficiency

15201600×100=95%\frac{1520}{1600}\times100=95\%

### Answer

- Frames = 52,632

- Overhead = 4,210,560 bytes

- Efficiency = 95%

### Effect

Higher protocol overhead reduces bandwidth efficiency and increases transmission time.

# 10. Hospital IoT Network

### Given

Original File

240 MB

Compression

30%

Segment Size

1400 bytes

Frame Overhead

60 bytes

### Compressed Size

240×0.70=168 MB240\times0.70=168\text{ MB}

Compressed Bytes

168000000168000000

### Segments

168000000/1400=120000168000000/1400=120000

### Overhead

120000×60=7200000 bytes120000\times60=7200000\text{ bytes}

= 7.2 MB

### Final Data

168000000+7200000=175200000 bytes168000000+7200000=175200000\text{ bytes}

= 175.2 MB

### Answer

- Compressed File = 168 MB

- Segments = 120,000

- Overhead = 7.2 MB

- Final Data = 175.2 MB

### Multiple OSI Layers

- Presentation Layer: Compresses and encrypts data.

- Transport Layer: Segments data, provides reliable delivery and flow control.

- Data Link Layer: Adds frame headers/trailers and performs error detection.

- Physical Layer: Transmits bits over the communication medium.

These solutions are suitable for 10-mark/15-mark university networking questions, showing the formulas, calculations, final answers, and brief theory.