SIMATS ENGINEERING

ASSESSMENT TOOL 3

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO4: 
Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards

(BL2,BL6)

Assessment Tool Weightage: 30%

QUESTIONS: 5								Total Marks:25

Annexure A

Comparative Analysis

Code of Conduct:

I __jeevika.k (192571060)__________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

Annexure A

Comparative Analysis

- Star topology is generally more expensive than bus topology because it requires a central device such as a switch or hub and more cabling to connect each node individually. In contrast, bus topology uses a single backbone cable, making it low-cost and easier to install. However, in terms of reliability, star topology is superior because failure of one node does not affect others, whereas a failure in the main bus cable can bring down the entire network. Performance is also better in star topology since data transmission is managed through a central device, reducing collisions, while bus topology suffers from performance degradation as more devices are added due to increased data traffic on the shared medium.

- Mesh topology offers very high fault tolerance because every node is connected to multiple other nodes, providing multiple paths for data transmission. If one link fails, data can still be routed through alternate paths without disrupting communication. In contrast, star topology has moderate fault tolerance; while failure of individual nodes does not affect the network, failure of the central hub or switch can cause the entire network to stop functioning. Therefore, mesh topology is more robust and reliable in critical environments compared to star topology.

- Bus topology is simple and easy to install because it uses a single backbone cable with minimal connections, making it suitable for small networks. On the other hand, mesh topology is highly complex to install as each node must be connected to every other node, requiring a large number of cables and ports. This complexity increases significantly as the number of devices grows, making mesh topology time-consuming and costly to implement compared to the straightforward setup of bus topology.

- Hybrid topology is highly scalable because it combines two or more different topologies (such as star, mesh, or bus), allowing the network to expand flexibly according to organizational needs. New segments can be added without affecting the existing network structure. In comparison, star topology is also scalable but to a limited extent, as it depends on the capacity of the central device (switch or hub). Once the central device reaches its port limit, expansion requires additional hardware. Thus, hybrid topology provides greater scalability than star topology.

- In terms of maintenance, star topology is relatively easy to manage because issues can be quickly identified and isolated to a specific node or cable. Mesh topology, while highly reliable, is difficult to maintain due to its complex structure and large number of connections, making troubleshooting time-consuming. Bus topology is harder to maintain than star because faults in the backbone cable can disrupt the entire network and are difficult to locate. Hybrid topology has moderate maintenance complexity; while it benefits from flexibility, managing multiple combined topologies requires careful monitoring and skilled administration.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 15% | Demonstrates complete and in-depth understanding of all concepts being compared | Good understanding with minor gaps | Basic understanding; some concepts unclear | Poor understanding of concepts |
| Comparison Depth | 20% | Provides detailed, insightful, and meaningful comparisons across multiple aspects | Adequate comparison with relevant points | Limited comparison; lacks depth | Minimal or incorrect comparison |
| Criteria Selection | 10% | Selects highly relevant and appropriate comparison parameters | Mostly relevant criteria | Some irrelevant or missing criteria | Poor or no criteria selection |
| Analytical Skills | 15% | Strong analysis with logical reasoning and clear justification | Good analysis with some justification | Basic analysis with weak reasoning | No clear analysis |
| Organization & Structure | 10% | Well-organized, clear, and logically structured | Generally organized | Somewhat disorganized | Poorly structured |
| Use of Examples / Evidence | 10% | Uses strong, relevant examples or data to support comparison | Uses some examples | Limited examples | No supporting examples |
| Clarity of Presentation | 10% | Very clear, concise, and easy to understand | Mostly clear | Somewhat unclear | Difficult to understand |
| Conclusion & Insights | 10% | Insightful conclusion with strong justification | Clear conclusion | Basic conclusion | No or weak conclusion |

# 1. Compare Star Topology and Bus Topology

| Star Topology | Bus Topology |
| --- | --- |
| Requires a central device such as a switch or hub. | Uses a single backbone cable. |
| Installation cost is high due to more cabling. | Installation cost is low because less cable is required. |
| Failure of one node does not affect other nodes. | Failure of the backbone cable causes the entire network to fail. |
| Better performance due to dedicated links. | Performance decreases as the number of devices increases. |
| Data collisions are minimal because communication passes through the switch. | Higher chance of data collisions because all devices share the same cable. |
| Easier to troubleshoot and maintain. | Difficult to locate faults in the backbone cable. |
| Suitable for medium and large organizations. | Suitable for small and temporary networks. |

### Conclusion

Star topology provides better performance, reliability, and fault isolation but has a higher installation cost. Bus topology is inexpensive and simple but is less reliable and performs poorly as the network grows.

# 2. Compare Mesh Topology and Star Topology (Fault Tolerance)

| Mesh Topology | Star Topology |
| --- | --- |
| Every node is connected to multiple nodes. | Every node connects to a central hub or switch. |
| Very high fault tolerance. | Moderate fault tolerance. |
| Multiple communication paths are available. | Only one path exists through the central device. |
| Failure of one link does not interrupt communication. | Failure of the central switch disconnects the entire network. |
| Suitable for mission-critical systems. | Suitable for office and educational networks. |
| More reliable. | Less reliable than mesh in case of central device failure. |
| Installation is expensive and complex. | Installation is simpler and less expensive. |

### Conclusion

Mesh topology offers superior fault tolerance and reliability, while Star topology provides easier management and lower cost.

# 3. Compare Bus Topology and Mesh Topology (Installation)

| Bus Topology | Mesh Topology |
| --- | --- |
| Uses one backbone cable. | Every node connects to every other node. |
| Very easy to install. | Difficult and time-consuming to install. |
| Requires fewer cables. | Requires a large number of cables. |
| Low installation cost. | High installation cost. |
| Suitable for small networks. | Suitable for critical applications. |
| Easy to expand initially. | Expansion becomes very complex. |
| Simple network design. | Complex network design. |

### Conclusion

Bus topology is simple and economical, whereas Mesh topology provides better reliability but requires a much more complex installation.

# 4. Compare Hybrid Topology and Star Topology (Scalability)

| Hybrid Topology | Star Topology |
| --- | --- |
| Combination of two or more topologies. | Uses one central switch or hub. |
| Highly scalable. | Moderately scalable. |
| New network segments can be added easily. | Expansion depends on available switch ports. |
| Supports large enterprise networks. | Best suited for medium-sized networks. |
| Flexible network design. | Less flexible than hybrid. |
| Supports different network technologies. | Supports only star-based structure. |
| Higher implementation cost. | Lower implementation cost. |

### Conclusion

Hybrid topology provides greater scalability and flexibility, while Star topology is easier and less expensive to implement.

# 5. Compare Maintenance of Different Network Topologies

| Topology | Maintenance |
| --- | --- |
| Star Topology | Easy maintenance because faults are isolated to individual nodes or cables. |
| Mesh Topology | Difficult maintenance due to many connections and complex wiring. |
| Bus Topology | Difficult to identify backbone cable faults, affecting the whole network. |
| Hybrid Topology | Moderate maintenance complexity because multiple topologies must be managed together. |

### Comparison Summary

| Feature | Star | Mesh | Bus | Hybrid |
| --- | --- | --- | --- | --- |
| Maintenance | Easy | Difficult | Moderate to Difficult | Moderate |
| Fault Detection | Easy | Complex | Difficult | Moderate |
| Reliability | High | Very High | Low | High |
| Cost | Medium | Very High | Low | High |
| Scalability | Moderate | Moderate | Low | Very High |

### Conclusion

- Star topology is the easiest to maintain.

- Mesh topology offers the highest reliability but is the most difficult to maintain.

- Bus topology is inexpensive but fault diagnosis is challenging.

- Hybrid topology balances flexibility and scalability, though it requires skilled administration.