SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO5: 
Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards

(BL2,BL5)

Assessment Tool Weightage: 40%

QUESTIONS: 3								Total Marks:30

Annexure A

Constraint-Based Problem Solving

Code of Conduct:

I __jeevika.k (192571060)__________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

Annexure A

Constraint-Based Problem Solving

- A  multinational company is opening a new branch office in a remote rural location where high-speed wired internet is unavailable. The office must support cloud-based applications, video conferencing, and secure communication with the headquarters. The company has a limited budget for connectivity but requires uninterrupted business operations. As a network engineer, recommend a suitable internet connection, networking devices, and security mechanisms. Justify your recommendations by considering bandwidth limitations, reliability, cost, and future scalability

- A university plans to provide campus-wide wireless internet access for more than 3,000 students across classrooms, laboratories, libraries, and hostels. The network must support a large number of simultaneous users while minimizing signal interference and ensuring secure access. Due to budget constraints, only a limited number of wireless access points can be deployed. Analyze the requirements and propose an efficient wireless network design, including access point placement, frequency band selection, and authentication methods. Explain how your solution balances coverage, performance, security, and cost.

- A financial institution is upgrading its internal computer network to improve cybersecurity while allowing employees to access sensitive customer information and online banking services. The organization must comply with strict security policies without significantly affecting network performance or increasing operational costs. Design a secure network architecture by selecting appropriate network segmentation techniques, firewall deployment, authentication methods, and intrusion detection mechanisms. Justify your design by considering the constraints of security, performance, compliance, and budget.

RUBRICS FOR CALCULATION

| Criteria | Weightage (%) | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Understanding of Problem & Constraints | 20% | Clearly identifies all constraints and fully understands the problem requirements | Identifies most constraints with minor gaps in understanding | Identifies some constraints but misses key aspects | Shows little or no understanding of constraints or problem |
| Problem Decomposition | 15% | Breaks problem into logical, well-structured sub-problems effectively | Breaks problem into sub-parts with some logical flow | Attempts decomposition but lacks clarity or structure | No clear decomposition of the problem |
| Application of Constraints in Solution | 20% | Applies all constraints correctly and consistently in solution design | Applies most constraints correctly with minor errors | Applies some constraints but with noticeable errors | Fails to apply constraints appropriately |
| Logical Reasoning & Strategy | 15% | Uses strong, clear, and efficient reasoning strategies | Uses generally sound reasoning with minor gaps | Reasoning is partially correct but inconsistent | Reasoning is unclear or incorrect |
| Solution Accuracy & Feasibility | 15% | Solution is fully correct, feasible, and optimized | Solution is mostly correct and feasible | Solution has errors or is partially feasible | Solution is incorrect or not feasible |
| Creativity & Innovation | 5% | Demonstrates highly innovative and unique approach | Shows some creativity in approach | Limited creativity shown | No creativity; relies on basic or copied ideas |
| Clarity of Presentation | 10% | Solution is clearly presented with excellent organization and explanation | Presentation is clear with minor issues | Presentation lacks clarity or organization | Poorly presented and difficult to understand |

# 1. Remote Branch Office Network Design

## Problem Statement

A multinational company is opening a branch office in a rural area where high-speed wired internet is unavailable. The network must support cloud applications, video conferencing, and secure communication while operating within a limited budget.

## Constraints

- No wired broadband connection

- Limited budget

- Reliable internet required

- Secure communication with headquarters

- Future network expansion

## Proposed Solution

### Internet Connection

- Primary: 5G/4G Fixed Wireless Access (FWA)

- Backup: Satellite Internet (if 5G coverage is unavailable)

Reason: Lower cost than leased lines, suitable for rural areas, and supports cloud services.

### Networking Devices

- 5G Router/Modem

- Enterprise Firewall

- Managed Gigabit Switch

- Wi-Fi 6 Access Point

- UPS (Power Backup)

### Security Mechanisms

- Site-to-Site VPN (IPSec VPN)

- WPA3 Wireless Security

- Firewall with Access Control Lists (ACL)

- Multi-Factor Authentication (MFA)

- Antivirus and Endpoint Protection

## Justification

| Constraint | Solution |
| --- | --- |
| Bandwidth | Use 5G broadband with QoS |
| Reliability | Backup satellite/secondary ISP |
| Cost | Avoid expensive leased lines |
| Security | VPN, Firewall, WPA3, MFA |
| Scalability | Additional APs and switches can be added easily |

## Advantages

- Cost-effective

- Secure communication

- Supports cloud applications

- Easy future expansion

- Reliable connectivity

## Conclusion

Using 5G Fixed Wireless Access, secure VPN connectivity, enterprise firewall, and WPA3 Wi-Fi provides a reliable, secure, and scalable network solution within the available budget.

# 2. University Campus Wireless Network Design

## Problem Statement

A university needs wireless internet for over 3,000 students across classrooms, laboratories, libraries, and hostels while using a limited number of access points.

## Constraints

- 3,000+ users

- Limited number of Access Points

- Budget limitations

- Secure access

- Low signal interference

## Proposed Wireless Design

### Access Point Placement

- One AP per classroom block

- One AP per laboratory

- Multiple APs in the library

- One AP on each hostel floor

- Ceiling-mounted APs for better coverage

### Frequency Band Selection

- 5 GHz: Classrooms and laboratories

- 2.4 GHz: Hostels and open areas

Reason:

- 5 GHz provides higher speed with less interference.

- 2.4 GHz offers wider coverage.

### Authentication Method

- WPA3 Enterprise

- IEEE 802.1X Authentication

- RADIUS Server

- Student login using university credentials

### Additional Optimization

- Enable Load Balancing

- Configure Fast Roaming (802.11r)

- Use Band Steering

- Assign non-overlapping Wi-Fi channels

- Enable Quality of Service (QoS)

## Justification

| Constraint | Solution |
| --- | --- |
| Coverage | Proper AP placement |
| Performance | 5 GHz + Load balancing |
| Security | WPA3 + RADIUS |
| Cost | Optimize AP locations instead of increasing AP count |
| Scalability | Easy addition of APs when needed |

## Advantages

- Good wireless coverage

- Reduced interference

- Secure authentication

- High performance

- Budget-friendly deployment

## Conclusion

A Wi-Fi 6 campus network with optimized AP placement, dual-band operation, WPA3 Enterprise, and RADIUS authentication provides secure, high-performance wireless access while minimizing deployment costs.

# 3. Secure Financial Institution Network

## Problem Statement

A financial institution requires a secure network to protect sensitive customer information while maintaining high performance and complying with security regulations.

## Constraints

- High cybersecurity

- Regulatory compliance

- High performance

- Controlled operational cost

## Proposed Secure Network Architecture

### Network Segmentation

Use Virtual LANs (VLANs)

- VLAN 10 – Management

- VLAN 20 – Banking Servers

- VLAN 30 – Employees

- VLAN 40 – Guests

- VLAN 50 – Data Center

### Firewall Deployment

- Next-Generation Firewall (NGFW)

- Internet Edge Firewall

- Internal Firewall between VLANs

- Web Application Firewall (WAF) for online banking

### Authentication

- Multi-Factor Authentication (MFA)

- Active Directory (AD)

- Role-Based Access Control (RBAC)

- Strong Password Policy

### Intrusion Detection

- Intrusion Detection System (IDS)

- Intrusion Prevention System (IPS)

- Security Information and Event Management (SIEM)

### Additional Security

- VPN for remote employees

- Data Encryption (TLS/SSL)

- Endpoint Antivirus

- Regular Security Audits

- Automatic Patch Management

## Justification

| Constraint | Solution |
| --- | --- |
| Security | VLANs, NGFW, IDS/IPS, MFA |
| Performance | VLAN segmentation reduces broadcast traffic |
| Compliance | Encryption, logging, SIEM |
| Budget | Use virtualization and centralized management |

## Advantages

- Strong protection against cyberattacks

- Improved network performance

- Regulatory compliance

- Secure online banking

- Scalable architecture

## Conclusion

A network architecture using VLAN segmentation, Next-Generation Firewalls, Multi-Factor Authentication, IDS/IPS, and encrypted communication provides a secure, high-performance, and scalable solution for a financial institution while meeting compliance and budget requirements.