SIMATS ENGINEERING

ASSESSMENT TOOL 3

COURSE NAME: Computer Networks			COURSE CODE:	CSA07

COURSE OUTCOME COVERED

CO5: Measure network performance using key metrics with simulation tools

Assessment Tool Weightage: 20%

QUESTIONS: 40								Total Marks:40

Annexure A

Mock Interview

Code of Conduct:

I _JEEVIKA.K (192571060)___________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

QUESTIONS:40						Total Marks:40

- Analyze how network congestion affects packet delay and packet loss. (BL4)(1)

- Compare the impact of star and mesh topologies on network reliability and fault tolerance. (BL4)(1)

- Why does increasing the number of routers between source and destination increase end-to-end delay? Explain logically. (BL4)(1)

- Analyze how Quality of Service (QoS) improves the performance of voice and video traffic. (BL4)(1)

- Evaluate the impact of using fiber optic cables instead of copper cables for high-speed communication. (BL5)(1)

- Analyze how network segmentation using VLANs improves performance and security. (BL4)(1)

- Compare the effects of TCP and UDP on network performance for different applications. (BL4)(1)

- Why is load balancing important in large enterprise networks? Analyze its impact on performance and availability. (BL5)(1)

- Analyze how frequent routing updates influence network efficiency and bandwidth utilization. (BL4)(1)

- Evaluate the trade-off between network security measures (such as firewalls and encryption) and overall network performance. (BL5)(1)

- A network has high delay—how will you identify and fix the issue? (BL4)(1)

- If throughput is low despite high bandwidth, what could be the reasons? (BL4)(1)

- How would you reduce packet loss in a congested network? (BL4)(1)

- A VoIP call is breaking frequently—how would you troubleshoot? (BL4)(1)

- Given a network with high jitter, suggest improvements (BL4)(1)

- How will you simulate a congested network in Packet Tracer? (BL5)(1)

- If packets are delayed in a router, what steps will you take? (BL4)(1)

- How would you optimize a network for online gaming? (BL4)(1)

- Design a small network and explain how you will measure its performance (BL5)(1)

- If RTT is very high, how will you diagnose the issue? (BL4)(1)

- Explain network performance to a non-technical person (BL5)(1)

- Describe throughput using a real-life example (BL5)(1)

- Present how packet loss affects video streaming (BL5)(1)

- Explain delay types clearly in under 1 minute (BL4)(1)

- Describe Cisco Packet Tracer in a professional way (BL5)(1)

- Explain QoS in simple and technical terms (BL5)(1)

- How would you present your simulation results to a client? (BL4)(1)

- Explain jitter using a real-world analogy (BL5)(1)

- Describe how you tested network performance in a lab (BL4)(1)

- Explain the importance of performance metrics in one structured answer (BL5)(1)

- Introduce yourself and explain your knowledge of network performance (BL5)(1)

- Why are you interested in networking and simulation tools? (BL5)(1)

- What makes you confident in analyzing network performance? (BL5)(1)

- Explain a concept you are very strong in (related to networking) (BL5)(1)

- How would you handle a question you don’t know? (BL5)(1)

- Describe a situation where you solved a technical problem (BL4)(1)

- Why should we select you for a networking role? (BL5)(1)

- What challenges did you face while learning Packet Tracer? (BL5)(1)

- How do you improve your technical skills regularly? (BL5)(1)

- Demonstrate confidence while explaining a complex concept simply (BL5)(1)

RUBRICS FOR CALCULATION:

| Criteria | Weightage (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Conceptual Understanding | 30% | Demonstrates thorough understanding; answers all questions accurately | Shows good understanding with few errors | Partial understanding; several incorrect responses | Lacks understanding; majority of answers incorrect |
| Accuracy of Answers | 25% | All answers are correct | Most answers are correct with minor mistakes | Some correct answers; frequent errors | Mostly incorrect answers |
| Application of Knowledge | 20% | Correctly applies concepts to problem-based questions | Applies concepts with minor errors | Limited ability to apply concepts | Unable to apply concepts |
| Time Management | 15% | Completes quiz well within time efficiently | Completes on time with minor delays | Requires extra time or rushes through questions | Unable to complete within given time |
| Question Interpretation | 10% | Interprets all questions correctly and responds appropriately | Misinterprets a few questions | Often misinterprets questions | Unable to understand questions |

# Network Performance – Questions and Answers

### 1. Analyze how network congestion affects packet delay and packet loss.

Answer: Network congestion occurs when too much data enters the network at the same time. Routers become overloaded, causing packets to wait in queues, which increases delay. If the queues become full, packets are dropped, resulting in packet loss. Therefore, congestion reduces network performance.

### 2. Compare the impact of star and mesh topologies on network reliability and fault tolerance.

Answer: In a star topology, failure of the central switch can affect the entire network, so its fault tolerance is lower. In a mesh topology, devices have multiple paths between them, so if one link fails, data can use another path. Therefore, mesh provides higher reliability and fault tolerance but is more expensive.

### 3. Why does increasing the number of routers between source and destination increase end-to-end delay? Explain logically.

Answer: Each router takes time to receive, process, queue, and forward packets. When more routers are present, these delays occur at every hop. Therefore, the total end-to-end delay increases as the number of routers increases.

### 4. Analyze how Quality of Service (QoS) improves the performance of voice and video traffic.

Answer: QoS gives priority to delay-sensitive traffic such as voice and video. It manages bandwidth and reduces delay, jitter, and packet loss. This results in clearer VoIP calls and smoother video streaming.

### 5. Evaluate the impact of using fiber optic cables instead of copper cables for high-speed communication.

Answer: Fiber optic cables provide higher bandwidth and faster data transmission than copper cables. They are also less affected by electromagnetic interference and can transmit data over longer distances. However, fiber is generally more expensive and requires specialized installation.

### 6. Analyze how network segmentation using VLANs improves performance and security.

Answer: VLANs divide a physical network into separate logical networks. This reduces unnecessary broadcast traffic and improves network performance. VLANs also isolate users and devices, improving security by limiting access between different network segments.

### 7. Compare the effects of TCP and UDP on network performance for different applications.

Answer: TCP provides reliable and ordered delivery using acknowledgements and retransmissions, but this creates more overhead and delay. UDP is faster and has lower overhead but does not guarantee delivery. TCP is suitable for web browsing and file transfer, while UDP is useful for gaming, streaming, and VoIP.

### 8. Why is load balancing important in large enterprise networks? Analyze its impact on performance and availability.

Answer: Load balancing distributes network traffic across multiple servers or links. It prevents one server or connection from becoming overloaded. This improves performance, response time, and availability and also provides better fault tolerance.

### 9. Analyze how frequent routing updates influence network efficiency and bandwidth utilization.

Answer: Routing updates help routers maintain accurate information about network paths. However, very frequent updates consume bandwidth and router processing power. Therefore, routing updates should be timely and efficient to maintain accurate routing without wasting network resources.

### 10. Evaluate the trade-off between network security measures such as firewalls and encryption and overall network performance.

Answer: Firewalls and encryption improve network security by filtering and protecting data. However, they require additional processing, which can slightly increase delay and reduce throughput. Therefore, security measures should be properly configured to achieve a balance between security and performance.

### 11. A network has high delay—how will you identify and fix the issue?

Answer: I would first use ping and traceroute to identify where the delay occurs. Then I would check bandwidth usage, congestion, router CPU utilization, and faulty links. I would fix the problem by removing bottlenecks, upgrading links, optimizing routing, or applying QoS.

### 12. If throughput is low despite high bandwidth, what could be the reasons?

Answer: Possible reasons include network congestion, packet loss, high latency, inefficient protocols, faulty hardware, interference, and server limitations. High bandwidth only represents the maximum capacity; actual throughput depends on the overall network conditions.

### 13. How would you reduce packet loss in a congested network?

Answer: I would identify the congested link and reduce unnecessary traffic. I could increase bandwidth, apply QoS, optimize routing, upgrade network devices, and use load balancing. Monitoring should then be used to confirm that packet loss has decreased.

### 14. A VoIP call is breaking frequently—how would you troubleshoot?

Answer: I would check packet loss, latency, jitter, bandwidth, and network connectivity. I would use ping and other network monitoring tools to locate the problem. Then I would prioritize VoIP traffic using QoS and fix congestion or faulty connections.

### 15. Given a network with high jitter, suggest improvements.

Answer: I would identify congestion and unstable network links. I would apply QoS to prioritize real-time traffic, reduce unnecessary traffic, improve bandwidth, and use better routing. These steps can reduce variation in packet arrival time.

### 16. How will you simulate a congested network in Packet Tracer?

Answer: I would create a network with multiple routers and generate traffic between different devices. I would configure limited-bandwidth links and send multiple data flows simultaneously. Then I would observe packet delay, packet loss, and network performance using simulation mode and network testing tools.

### 17. If packets are delayed in a router, what steps will you take?

Answer: I would check the router's CPU and memory usage, interface status, queue size, bandwidth, and routing table. I would also check for congestion and packet errors. After identifying the cause, I would optimize routing, increase bandwidth, or apply QoS.

### 18. How would you optimize a network for online gaming?

Answer: I would reduce latency, jitter, and packet loss by using a stable high-speed connection. I would prioritize gaming traffic using QoS, avoid unnecessary background traffic, use efficient routing, and connect through Ethernet when possible.

### 19. Design a small network and explain how you will measure its performance.

Answer: I would connect several PCs to a switch, connect the switch to a router, and connect the router to the Internet.

Performance measurements:

- Latency: Use ping.

- Packet loss: Send multiple ping packets and calculate lost packets.

- Throughput: Measure the amount of data transferred per second.

- Jitter: Compare the variation in packet arrival times.

- Bandwidth utilization: Monitor link usage.

### 20. If RTT is very high, how will you diagnose the issue?

Answer: I would use ping to measure RTT and traceroute to identify the problematic hop. Then I would check congestion, routing paths, bandwidth, overloaded devices, and physical links. I would optimize routing or remove the source of congestion.

### 21. Explain network performance to a non-technical person.

Answer: Network performance is like the performance of a road. If the road is wide and traffic is low, vehicles move quickly. Similarly, a good network allows data to move quickly with low delay and low packet loss. Important measures include speed, delay, and reliability.

### 22. Describe throughput using a real-life example.

Answer: Throughput is the actual amount of data successfully transferred in a given time. For example, a water pipe may have a maximum capacity of 100 litres per minute, but if only 70 litres actually pass through, the actual flow is 70 litres per minute. Similarly, network throughput is the actual data transferred per second.

### 23. Present how packet loss affects video streaming.

Answer: Packet loss means some video data does not reach the destination. This can cause buffering, freezing, poor video quality, or missing parts of the video. Higher packet loss generally results in a worse streaming experience.

### 24. Explain delay types clearly in under 1 minute.

Answer: There are four main types of network delay. Processing delay is the time a device takes to process a packet. Queuing delay is the time a packet waits in a queue. Transmission delay is the time required to place the packet onto the link. Propagation delay is the time taken for the signal to travel through the medium.

### 25. Describe Cisco Packet Tracer in a professional way.

Answer: Cisco Packet Tracer is a network simulation and learning tool developed by Cisco. It allows users to design, configure, and troubleshoot virtual networks without requiring physical hardware. It is useful for learning networking concepts, testing configurations, and analyzing network behavior.

### 26. Explain QoS in simple and technical terms.

Answer: Simply, QoS is like giving an ambulance priority on a busy road. Technically, Quality of Service manages network traffic by prioritizing important applications such as VoIP and video. It helps control bandwidth, delay, jitter, and packet loss.

### 27. How would you present your simulation results to a client?

Answer: I would present the network design, testing method, and key performance metrics such as throughput, latency, packet loss, and jitter. I would use simple graphs or tables to show the results and clearly explain problems, improvements, and recommendations.

### 28. Explain jitter using a real-world analogy.

Answer: Jitter is like a bus that should arrive every 10 minutes but sometimes arrives after 5 minutes and sometimes after 20 minutes. The timing is inconsistent. Similarly, jitter is the variation in the arrival time of network packets.

### 29. Describe how you tested network performance in a lab.

Answer: In the lab, I created a network using PCs, switches, and routers. I tested connectivity using ping and checked the path using traceroute. I measured delay, packet loss, and throughput under different traffic conditions and compared the results.

### 30. Explain the importance of performance metrics in one structured answer.

Answer: Network performance metrics help us understand how efficiently a network operates. Bandwidth shows capacity, throughput shows actual data transfer, latency shows delay, packet loss shows reliability, and jitter shows variation in delay. These metrics help identify problems and improve network performance.

## Personal / Interview Questions

### 31. Introduce yourself and explain your knowledge of network performance.

Answer:
I am a networking student with a strong interest in computer networks and network performance. I have learned concepts such as bandwidth, throughput, latency, packet loss, jitter, QoS, and network topologies. I also have practical experience using Cisco Packet Tracer to design and analyze basic networks. I am interested in improving my practical networking skills.

### 32. Why are you interested in networking and simulation tools?

Answer:
I am interested in networking because it explains how computers and devices communicate with each other. Simulation tools such as Cisco Packet Tracer allow me to practice network configuration without requiring physical equipment. They help me understand concepts practically and improve my troubleshooting skills.

### 33. What makes you confident in analyzing network performance?

Answer:
I understand the basic performance metrics and how they affect network communication. I can use tools such as ping, traceroute, and Packet Tracer to identify problems. I also follow a systematic approach by identifying the problem, analyzing its cause, applying a solution, and testing the result.

### 34. Explain a concept you are very strong in related to networking.

Answer:
I am particularly confident in understanding network topologies. A topology describes how devices are connected in a network. For example, star topology uses a central switch, while mesh topology provides multiple connections between devices. Understanding topology helps in selecting networks based on cost, reliability, and performance requirements.

### 35. How would you handle a question you don’t know?

Answer:
I would honestly say that I do not know the answer instead of giving incorrect information. I would explain what I know about the related concept and then try to find the correct solution through documentation, testing, or guidance. I see unknown questions as opportunities to learn.

### 36. Describe a situation where you solved a technical problem.

Answer:
During a networking lab, I faced a situation where two devices could not communicate. I checked the physical connections and IP addresses first. Then I checked the subnet configuration and gateway settings. After correcting the configuration, I tested the connection using ping and successfully restored communication.

### 37. Why should we select you for a networking role?

Answer:
You should select me because I have a strong interest in networking, a willingness to learn, and a basic practical understanding of network configuration and performance analysis. I am comfortable learning new technologies and solving problems systematically. I am also committed to continuously improving my technical skills.

### 38. What challenges did you face while learning Packet Tracer?

Answer:
Initially, I found IP addressing, router configuration, and troubleshooting difficult. I overcame these challenges by practicing different network scenarios and checking configurations step by step. Regular practice helped me become more comfortable with Packet Tracer.

### 39. How do you improve your technical skills regularly?

Answer:
I improve my technical skills through regular practice, lab exercises, online learning, and small networking projects. I use simulation tools such as Packet Tracer to test different configurations. I also review concepts and try to understand how they work in real-world situations.

### 40. Demonstrate confidence while explaining a complex concept simply.

Answer:
A complex networking concept can be explained using a simple real-life example. For example, QoS can be compared to an ambulance getting priority on a busy road. In a network, important traffic such as voice and video gets higher priority so that it reaches the destination with less delay and interruption. This makes the concept easy to understand while still explaining its technical purpose.